How to Export DICOM Files from Planmecca

- Anywhere on your desktop, Right click → select New → select Folder and name the new folder with the Patient’s name

- Open the patient’s scan in Romexis and select the volume you want to export and load the scan

- Go to OUTPUT and click on EXPORT VOLUME

- A 3D export window should now appear. Continue to select the following settings:

- Under File and format options: Set of single frame DICOM files

- Resolution: Original

- Overlay options: (No check marks)

- Export 3D Volume to: Folder

- Viewer Options: (No check marks. Viewer is not needed)

- Next to folder on the right, Select “…” Look for the folder you created on your desktop with the patient’s name. Select this as your export location

- File name: Please include the Patient Name, Study Type information (example: Mandible) and date. Labelling is very beneficial, so include this information if possible.

- Select “OK” to export the DICOM Files

- After the files have been exported, locate the file you created earlier with the patient’s name.

- Right click on the file, click on “send to” → “Compressed (zipped) folder”

- This step compresses the individual DICOM files into a single .zip file that you can upload to the patient’s case on the O-RAD website.